###### TIER 1 LOOKUP TABLE 2008

|  | Groundwater (mg/L) | Soil (mg/kg) Sensitive Area |  | Soil (mg/kg) Non-Sensitive Area |  | Soil (mg/kg) | Soil (mg/kg) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical of Concern |  | Residential Surface Soil | 1 Leaching to GW | Non-residential Surface Soil | 1 Leaching to GW | Oil Mobility Limit | 2Plant Protection Limit |
| Benzene | 0.005 | 44 | 3 | 190 | 3 | NA | NA |
| Toluene | 1 | 6700 | RES | 52000 | RES | NA | NA |
| Ethylbenzene | 0.7 | 5400 | RES | 60000 | RES | NA | NA |
| Xylenes | 10 | 21000 | RES | 100000 | RES | NA | NA |
| Total Petroleum Hydrocarbons, condensate | 5 | 4400 | RES | 46000 | RES | 3000 | 10000 |
| Total Petroleum Hydrocarbons, crude oil | 5 | 2600 | RES | 41000 | RES | 13000 | 10000 |
| RES = Target hazard index could not be reached at any concentration of this chemical. |  |  |  |  |  |  |  |
| NA = Not Applicable |  |  |  |  |  |  |  |
| 1 Leaching to GW assumes that the receptor well is located beneath the source. To consider alternate well locations downgradient from the source (Tier 1a), use the Oklahoma Spreadsheet for Calculating Risk Based Screening Levels to calculate the Tier 1a RBSL. 2 The Plant Protection Limit only applies in the root zone (see glossary, Appendix V). |  |  |  |  |  |  |  |

Use the CSM for each site to determine which exposure pathways are complete.  When using the Tier I Lookup Table to determine the appropriate risk-based screening level for a specific chemical or substance at a site, use the strictest (lowest) RBSL for all complete exposure pathways or the mobility limit, whichever is lower.  For example, when considering a condensate spill in a non-residential area, 46,000 mg/l is the risk-based number, but 10,000 mg/l is the general plant protection limit and 3,000 mg/kg is the soil mobility limit.  3,000 mg/kg is the number that must be used. Benzene must also be analyzed at a condensate spill site.  The appropriate RBSL to meet for benzene is 190 mg/kg, if leaching to groundwater is not a complete exposure pathway at the site. However, if leaching to groundwater is a pathway of concern at the site, then the appropriate RBSL is 5 mg/kg.